# Kitchen/Nook

FLOOR:

- Remove old vinyl tiles (2 layers) & tar paper

- Prep & level hardwood floor

- Refinish or resurface per customer specifications

WALLS:

- Remove old “tile” sheeting

- Strip wallpaper

- Repair/sand plaster walls as necessary

- Tile (or other) backsplash & stove areas

- Repaint/repaper per customer specifications

CEILING:

- Repair cracks & repaint

WINDOWS:

- 4 windows in working condition

- new triple track storms/screens installed

- replace window hardware

- replace 3 counterweights (east windows)

- Replace 1 sill by sink area

- Refinish windows & sills

ELECTRICAL:

- 1 baseboard outlet

3 round outlets in misc. places

1 outlet under North windows

- Fluorescent light fixtures in both kitchen & nook

- doorbell over dining room door

GAS/HEAT:

- Radiator in SW corner of nook

- Gas line in NW corner of kitchen

- Gas line on S wall of kitchen

CABINETS:

- Remove & replace sink & countertops

- Remove built-in cabinets on S & E walls

- Remove built in cabinet on W walls of nook & kitchen

- Install new cabinets & countertops per customer specifications

PLUMBING:

- Sink on E wall of kitchen

MISC:

- Corner shelves in SE corner of nook

- Baseboards good condition—repaint

- Doors & frames in good condition—repaint

- New shoe mold needed

# DINING ROOM

FLOOR:

- Sand & refinish hardwood floor

WALLS:

- Strip wallpaper

- Repair/sand plaster walls as necessary

- Repaint/repaper per customer specifications

CEILING:

- Repair cracks & repaint

- Possible water damage near living room entry

WINDOWS:

- 3 functional windows on W wall

1 functional window on N wall

- new triple track storms/screens installed

- replace window hardware

- Refinish windows & revarnish to original condition

ELECTRICAL:

- 2 outlets in baseboards:

1 on S wall

1 on N wall

- Overhead light fixture—switch next to kitchen door

- 1 oddball TV/radio outlet on S wall

GAS/HEAT:

- Radiator in NE corner

CABINETS:

- Built-in cabinets on W wall with leaded glass doors

- Window seat—refinish & replace rotted wood sections

- Repaint interior of cabinets

- 1 broken pane in NW cabinet door

MISC:

- Baseboards good condition—refinsh/revarnish

- Doors & frames in good condition—refinish/revarnish

- New light fixture?

# LIVING ROOM

FLOOR:

- Sand & refinish hardwood

WALLS:

- Strip wallpaper

- Repair damage above S windows

- Repair/sand plaster walls as necessary

- Repaint/repaper per customer specifications

CEILING:

- Remove acoustic tiles & crown molding

- Repair cracks & repaint (possible drywall?)

WINDOWS:

- 2 fixed windows on W wall

3 windows on S wall, one center window currently functional, outside windows painted shut

- replace window hardware

- Refinish windows & sills

ELECTRICAL:

- 2 outlets on N wall

2 on E wall

1 on S wall

- 2 oddball TV outlets on S wall

- Phone jack on S wall

- Coax cable from SW corner

- No overhead or built-in light fixtures

- Florescent fixture in fireplace mantle

- Thermostat on N wall by dining room entry

GAS/HEAT:

- Radiator under S windows with large window seat

- Brick fireplace on W wall:  good condition (clean only?)

- Newer brass doors on fireplace

- Update fireplace mantle & brick face

CABINETS:

- Built-in/free standing shelves on W wall next to fireplace—baseboard trimmed to fit

- Large radiator enclosure under S windows

MISC:

- Strip & refinish archways & windows

- Crown moldings rough

- shoe mold rough

# FRONT DOOR/FRONT HALL/STAIRS

FLOOR:

- Remove old vinyl floor tiles

WALLS:

- Strip wallpaper

- Repair/sand plaster walls as necessary

- Repaint/repaper per customer specifications

CEILING:

- Repair cracks & repaint

- Crown molding rough

WINDOWS:

- 1 functional window on E wall—replace 1 counterweight

- Fixed windows on either side of front door

- Strip & refinish as necessary per customer specifications

- New hardware

ELECTRICAL:

- No outlets

- Replace overhead fixture

- Unknown switch near living room entry

STEPS:

- Strip & refinish stairs & balusters per customer

- Hand rail added—not original

- Closet door has large mirror

FRONT DOOR:

- Refinish per customer specifications

MISC:

- Baseboards & shoe mold good—refinish

# FRONT DOOR/FRONT HALL/STAIRS

FLOOR:

- Remove vinyl flooring

- Repair as necessary

- Refinish per customer specifications

WALLS:

- Brick—repaint

CEILING:

- Strip paint

- Repair/replace wood as necessary

- refinish as original (varnished)

WINDOWS:

- 9 windows with newer triple-track storm windows

West walls has original swing-out windows

N wall has 3 windows from living room

ELECTRICAL:

- 2 overhead light fixtures—1 switched, 1 with extension cord into living room

- No outlets

- Extension cord from living room through N window sill

MISC:

- Shoe mold poor condition

- Concrete step into house needs paint

- Floor angles down towards street

# STAIR LANDING/STAIRS TO 2ND FLOOR

FLOOR:

- Refinish per customer specification

WALLS:

- Strip wallpaper

- Repair/sand plaster walls as necessary

- Repaint/repaper per customer specifications

- 1 newer handrail added to stairs

CEILING:

- Repair cracks & repaint

- Crown molding rough—poor fit

WINDOWS:

- 2 functional windows on E wall—left window needs new counterweights

ELECTRICAL:

- 1 covered outlet in S wall baseboard

STEPS:

- Strip & refinish stairs & balusters per customer

- Hand rail added—not original

MISC:

- Sliding pocket door in S wall

- Baseboard good, shoe mold poor

# UPSTAIRS HALL

FLOOR:

- Water damage to hardwood @ bathroom door

- Repair & refinish as necessary

WALLS:

- Strip wallpaper

- Repair/sand plaster walls as necessary

- Repaint/repaper per customer specifications

CEILING:

- Good condition—repaint only

ELECTRICAL:

- Light fixture on ceiling, switch on W wall and in downstairs front hall

- Phone jack on W wall

CABINETS:

- 3 drawers & cabinet built into E wall

- Refinish per customer

- new hardware

MISC:

- Crown molding fits poorly

- Shoe molding rough

- baseboards OK

- Refinish all woodwork per customer

# UPSTAIRS SUNROOM/PORCH

FLOOR:

- Strip & repair water damage to hardwood

- Refinish

WALLS:

- Strip wallpaper

- Repair damaged plaster under windows on N wall

- Refinish per customer specifications

CEILING:

- Serious water damage—locate water source & repair

- Patch hole in ceiling

- Refinish per customer specifications

WINDOWS:

- 8 functional windows

- Window frames & sills already stripped

- Refinish per customer

- New hardware

ELECTRICAL:

- 1 outlet in S wall baseboard

- Old-style phone jack on S wall

HEAT:

- Radiator on W wall under windows

CABINETS:

- Shelves screwed into S wall

MISC:

- Ceiling appears to have been repaired once before

- Some water damage to window frames on N wall

# NE Bedroom (possible master suite)

FLOOR:

- Refinish hardwood—several different stains used in the past

WALLS:

- Strip wallpaper

- Repair damaged plaster under windows on N wall

- Repair cracks in closet plaster

- Refinish per customer specifications

CEILING:

- Repair cracks in plaster

- refinish per customer specifications

WINDOWS:

- 1 functional window on E wall

ELECTRICAL:

- 1 outlet on N wall

1 outlet on E wall

- Ceiling fixture with switch by hall door

HEAT:

- Radiator in NW corner

- Gas line near hall door

CABINETS:

- Shelves built into S wall, poorly made

MISC:

- Strip & refinish doors

- odd crown molding gap @ ceiling

- Baseboards & shoe mold OK

- Chimney intrusive—remove?

# SE Bedroom (master?)

FLOOR:

- Refinish hardwood

WALLS:

- Strip wallpaper

- Repair damaged plaster under windows on N wall

- Repair cracks in closet plaster

- Refinish per customer specifications

CEILING:

- Good condition, recently resurfaced (textured)

WINDOWS:

- 2 functional windows in S wall

- Strip & refinish per customer

- New hardware

- Window frames & sills OK

ELECTRICAL:

- Covered outlets in N, E & W walls

- Ceiling fan installed, switch @ door

- Pull chain light in SW closet

- Phone jack on E wall

HEAT:

- Radiator in SW corner

MISC:

- Mirror on door of SW closet

- All moldings OK

- Improvised storage in NE closet

# UPSTAIRS BATH

FLOOR:

- Remove vinyl flooring

- Prep subfloor/repair water damage & rotten boards

- Resurface & refinish per customer specifications

WALLS:

- Remove sheeting

- Remove tile around tub

- Repair plaster as necessary

- Refinish per customer specifications

CEILING:

- Remove crown molding

- Remove sheeting

- Repair cracks

- Refinish per customer specifications

WINDOWS:

- 1 functional window on W wall

ELECTRICAL:

- 1 outlet on N wall, next to medicine cabinet

- 2 fluorescent fixtures next to medicine cabinet—no switch, activated by pull chains

- Install exhaust fan, new lighting with wall switches

HEAT:

- Radiator on W wall

CABINETS:

- Medicine cabinet on N wall

PLUMBING:

- Sink on N wall—replace

- Toilet & bathtub on S wall—replace both

- Build shower and/or shower/tub

- Replace pipes as necessary

- Toilet may be recent

MISC:

- Baseboard OK, needs paint

- Replace shoe molding

- Refinish door & door frame per customer specifications

# SW BEDROOM

FLOOR:

- Repair water damage, determine leak source & repair if necessary

- Refinish floor

WALLS:

- Very poor condition—repair cracks and repaint or repaper

- Plumbing access door in N wall

CEILING:

- Recently refinished with textured patter

- Serious cracks returning—repair properly

- Refinish ceiling per customer specifications

WINDOWS:

- 2 functioning windows on S wall

1 functioning on W wall

- Frames & sills OK

- Refinish per customer specifications

ELECTRICAL:

- Outlets in all 4 walls

- Coax cable from within W wall

- Light fixture on ceiling, switch by hall door

- Closet has pull-chain light fixture

HEAT:

- Radiator in SE corner

CABINETS:

- Crude shelves built into closet

- Shelves hung on E wall

MISC:

- Nice wallpaper in the closet, man!

- Mirror on closet door

- All moldings fair condition

# NW Bedroom (possibly to be annexed for master suite)

FLOOR:

- Refinish hardwood

WALLS:

- Strip wallpaper

- Repair cracks in plaster

- Repair cracks in closet plaster

- Refinish per customer specifications

CEILING:

- Good condition—repair cracks in plaster

- refinish per customer specifications

WINDOWS:

- 1 functional window on N wall

1 functioning on W wall

- West window needs new counterweight

- Possible water damage on W window frame

- Strip & refinish per customer specifications

ELECTRICAL:

- 1 outlet on N wall

1 outlet on W wall

- Ceiling fixture with switch by hall door

- No light in closet—interesting design on ceiling

HEAT:

- Radiator in NE corner

MISC:

- Mirror in closet door

- Moldings OK—again strange gap between ceiling & crown molding

# SIDE ENTRY/DOWNSTAIRS HALL

FLOOR:

- Remove Linoleum floor

- Repair as necessary

- Refinish per customer specifications

WALLS:

- Remove wall paper

- repair & refinish plaster

- 2 balusters, 1 original, 1 newer

CEILING:

- Repair small cracks & repaint—heavy damage in entry area

ELECTRICAL:

- Overhead pull chain lights in hall & above side door

- Switch at top of stairs for basement

- Switch next to side door for outside light

- No outlets

MISC:

- Coat rack by side door

- coat rack in hall by kitchen

- Extra hand rail on stairs

- Newer side door & screen

# BASEMENT—MAIN ROOM, BASE OF STAIRS

FLOOR:

- Rough concrete, water damage present

- Strip (muriatic acid?) & repaint

WALLS:

- Brick walls water damaged, paint peeling

- Wood walls moldy with some water damage

- Skim coat concrete over brick?  Other barrier?

- Strip, repair, repaint wood walls

CEILING:

- Poor condition

- Repair rotted section—some repaired once before

- Resurface & repaint OR install drop ceiling

WINDOWS:

- 2 windows on N wall—fixed in place

ELECTRICAL:

- 3 pull chain ceiling fixtures, 1 controlled from top of stairs

- 220 outlet on N wall

- 2 standard outlets on N wall

- Breaker boxes on N wall

PLUMBING:

- Some appears recently repainted or replaced

- Shower fixture on E wall

- 3 tub sink unit with spigot on N wall with washer hook-up

- Drain in floor @ E end by shower

- Repair/upgrade as necessary

# BASEMENT—WORK ROOM

FLOOR:

- Concrete, fair condition

- Strip & repaint

- Built-up under furnace

WALLS:

- Brick walls water damaged, paint peeling

- Wood walls moldy with some water damage

- Skim coat concrete over brick?  Other barrier?

- Strip, repair, repaint wood walls

CEILING:

- Poor condition—extensive water damage

- Repair rotted section—some repaired once before

- Resurface & repaint OR install drop ceiling

WINDOWS:

- 2 windows in W wall, 1 in E wall

- Fair condition—scrape & repaint

ELECTRICAL:

- 3 pull-chain lights, 1 fixture with switch

- 1 outlet on E wall

- Install florescent fixtures instead

- Main power on S wall

PLUMBING:

- Water meter on S wall

- Hot water heater near W wall—appears to be leaking from base

HEAT:

- Furnace in center of room—newer

- Chimney on W wall

CABINETS:

- Built-in shelves in W wall

- Racks on E wall

- Large workbench on S wall—may as well be built-in

# COAL ROOM—UNDER FRONT PORCH

FLOOR:

- Brick floor, good condition

WALLS:

- Brick & cinderblock walls—decent condition

CEILING:

- Open to floor joists—good condition

ELECTRICAL:

- 1 pull chain light fixture

MISC:

- Gas meter on S wall

- Coal door needs securing

# BASEMENT—BATHROOM

FLOOR:

- Rough concrete, water damage present

- Strip (muriatic acid?) & repaint

WALLS:

- 3 sides wood, 1 brick

- Resurface brick wall, repaint wood, strip existing wallpaper

CEILING:

- Poor condition—drooping

- Repair & refinish plaster

WINDOWS:

- 1 window on W wall—fixed in place

ELECTRICAL:

- 1 overhead pull chain

PLUMBING:

- Toilet—upgrade or replace?

- Sink—remove & replace

- New hardware

# PANTRY AREA

- Remove wall between main room & pantry

- Paint floor

- Fuse box on N wall

- Shelves on W & E wall

- Window on N wall

- Hanging light fixture with pull chain.

# ATTIC—MAIN ROOM, TOP OF STAIRS

FLOOR:

- Remove carpet & Linoleum

- Lay new carpet—not hardwood

WALLS & CEILING:

- Remove cardboard panels

- Hang & finish new drywall

- Paint or wallpaper per customer specifications

WINDOWS:

- 3 functional windows on S wall, 1 on E wall

- Strip & refinish per customer specifications

ELECTRICAL:

- 1 switched fixture over stairs, 2 pull chain fixtures in main area, 1 wall-mount fixture on E wall

- 1 electric outlet on W wall

- phone jack on E wall by stairs

- Install ceiling fan in cathedral area

HEAT:  N/A

PLUMBING:  N/A

CABINETS:

- 6 drawers & 2 doors in N wall

- Sand & refinish per customer specifications

# ATTIC—SMALL ROOM

FLOOR:

- Remove Linoleum

- Install new flooring (carpet or tile?)

WALLS:

- Strip wallpaper

- Repair wallboard or replace with drywall

CEILING:

- Textured wall board—remove & replace with drywall

- Install new crown molding

WINDOWS:

- 2 functional windows on W wall

- Replace all counterweights in 2 both windows

- Refinish with new hardware

ELECTRICAL:

- pull-chain fixture

- fixture on S wall

- No outlets

HEAT:

- Radiator in NW corner

MISC:

- Closet OK as-is (wallboard good condition)

- Moldings all OK